KFLDTV – TV - Kennfeld für LDR

KFLDTV = Duty Cycle Start Map for Boost Control / Boost Control Start WG Duty Cycle Map

X / Y / Z = Throttle Angle [degrees] / RPM / Percentage (Prozent [%])

This is Pre-PID or Boost Control Start WG Duty Cycle Map, where boost control starts at defined wastegate duty cycle to reach the desired or requested boost (load in old Motronics)

From ME7:

KFLDRL - Map for linearization of boost pressure = f(TV). This is the post-PID waste-gate duty correction table. This is the N75 duty lookup table before PID corrections take place. These values may need altered depending on wastegate base spring pressure. Calibrating this correctly is time consuming, but worth the effort.

KFLDRL can also be used to get open-loop type behavior for operation past the MAP and requested boost limit by making the output duty cycle unresponsive (flat) to uncorrected duty cycle (from the PID) at various RPM/DC points.

With aftermarket or external waste-gates, flat line the map at something like 25% until your turbo should be spooled. Then taper off to 10% at areas of higher load and RPM. Log requested vs. actual boost to see where you need to adjust KFLDRL to line things up. The higher your aftermarket waste-gate spring pre-load the lower the WGDC you will need to accurately control boost. You may need to numb KFLDRL response a bit since very tight wastegates need very little DC change to make a big difference in boost.